Alphabet of the Assembler

Categories of Commands

- Arithmetic and Logical (ALU Commands)

- Shift Commands

- Memory Commands

- Branch Commands

- Jump and Subroutine Commands

- Immidiate Commands

- Arithmetic and Logical (ALU Commands)

| OPCODE “0000” |  |
| --- | --- |
| ADD | 000001 |
| SUB | 000010 |
| AND | 000011 |
| OR | 000100 |
| XOR | 000101 |
| NOT | 000111 |
| OTHER BITS | DON’T CARE |

- Shift Commands

| OPCODE “0001” |  |
| --- | --- |
| SHL | 000001 |
| SHR | 000010 |
| ROL | 000011 |
| ROR | 000100 |

- Memory Commands

| OPCODE “ 0010“ |  |
| --- | --- |
| LOAD | 000001 |
| STORE | 000010 |

- Branch Commands

| OPCODE “0011” |  |
| --- | --- |
| BEQ | 000001 |
| BNE | 000010 |
| BLT | 000011 |
| BGT | 000100 |

- Jump and Subroutine Commands

| OPCODE “0100” |  |
| --- | --- |
| JMP | 000001 |
| CALL | 000010 |
| RET | 000011 |

- Comparison commands

| OPCODE “0101” |  |
| --- | --- |
| CMP | 000001 |

- Immediate Commands

| OPCODE “0110” |  |
| --- | --- |
| MOVI | 000001 |

OPERANDS

Operands include registers, immediate values, and memory addresses.

- REGISTERS

Symbol: R0, R1,…, R27 (32 registers, encoded as 5-bit values).

BINARY ENCODINGS

| GENERAL PURPOSE |  |
| --- | --- |
| R0 | 00000 |
| R1 | 00001 |
| R2 | 00010 |
| R…. | …… |
| R27 | 11011 |

| SPECIAL PURPOSE |  |
| --- | --- |
| PC (R28) | 11100 |
| R-MEM (R29) | 11101 |
| LR (R30) | 11110 |
| FR (R31) | 11111 |

SYNTAX RULES

GENERAL SYNTAX:

COMMAND OPERAND1, OPERAND2, OPERAND3

COMMAND OPERAND1, OPERAND3

- COMMAND: The operation mnemonic (e.g., ADD, BEQ, MOVI).

- OPERAND1: Always register (source 1).

- OPERAND2: Immidiate value, Register or Mem Address (source 2).

- OPERAND3: Destination (Reg or Mem Address)

SINGLE ENDED COMMANDS:

- NOT R1, R2 (Invert R1 and store result in R2)

- RET (Return from subroutine).

BRANCH AND JUMP COMMANDS:

- JMP LABEL_NAME

- BEQ LABEL_NAME (based on CMP command results)

IMMIDIATE COMMANDS:

- MOVI R1, #10 (Move 10 into R1).

LABEL DEFINITION:

- LABEL_NAME: (Define a label for a memory address).

IMMIDIATE AND MEM VALUES VALUES:

- Before the immidate value ther is #

Example #20 immidiate value

- Mem address must be between” [“,”]” symbols

  - Example LOAD R1, [0x1000]

COMMENTS:

Comments start with a semicolon (;) and are ignored by the assembler.

Example`

; Program example

MOVI R1, #5                                        ; Load 5 into R1

Parsing Rules

Line Structure:

- A line can contain:

  - A label (LABEL:).

  - An instruction (COMMAND OPERAND1, OPERAND2, OPERAND3).

  - A comment (;).

EXAMPLE:

- START:					           // label

; This is the beginning of the program     // comment

ADD R1, R2, R3		  	           // command

CMP R1,R2

Whitespace:

- Lines may include spaces or tabs for alignment.

- Extra whitespace should be ignored.

Case Insensitivity:

- Commands and operands should be case-insensitive (ADD is equivalent to add).

Example Assembly Code:

; Program to add two numbers

START:  ; label

MOVI R1, #10                      ; Load 10 into R1

MOVI R2, #20                      ; Load 20 into R2

ADD  R1, R2, R3                  ; R3 = R1 + R2

STORE R3, MEM[0x0100]                            ; Save result in memory at 0x0100

JMP START                         ; Repeat without conndition

struct Instruction

{

std::string istr_type // ALU SHIFT MEM and so on – 4 bit

std::string opcode; // Command mnemonic, e.g., ADD, SUB – 6bit

std::string source1; // Source 1 operand (always register) – 5 bit

str::string source_2_type; // can be register immediate or mem addres – 2 bit

std::string source2; // Source 2 operand (register/immediate/memory) – 16 bit(if reg 5LSBs)

std::string dest_type; // Destination type (reg, mem)  - 1bit

std::string destination; // Destination operand (register/memory)  16 bit if reg use 5LSBS

bool requires_simplification = false; // Flag for memory operations requiring LOAD };